# Slope Stability Analysis

Slope Stability Analysis Report

| Date: | October 10, 2026 |
| --- | --- |

Table of Contents

1 Traceability

2 Project Definition

3 Limit Equilibrium Analysis

3.1 Analysis Inputs

3.2 Materials

3.3 Loads

3.4 Spencer's Method

3.4.1 Search for the Critical Surface

3.4.2 Results

3.4.3 Slice Table

3.4.4 Calculations

Page numbers appear when the table is updated in Word (right-click → Update Field, or select all and press F9).

# Traceability

The results in this report were computed by xslope from the input file below. Its SHA-256 digest identifies that file exactly.

| xslope version | 0.5.2 |
| --- | --- |
| Input file | xslope_simple_mult_layers.xlsx |
| Input file SHA-256 | c571717be7703ee3a36aa8a674fb3ca1654b507034d2b1b288213f3c95179945 |
| Report generated | 2026-10-10 23:48 |

# Project Definition

The problem cross section is defined by 2 material zones described with profile lines. The zones are named embankment and foundation. The problem definition is displayed in Figure 1, including the geometry, the material zones and the maximum depth line the zones are closed at.

All quantities are in US customary units: lengths in ft, stresses and pressures in psf, unit weights in pcf, and forces per unit thickness of section in lb/ft. Angles are in degrees. The analysis is two-dimensional: every force is per unit thickness normal to the section. The unit weight of water is 62.4 pcf.

The materials the analysis reads off this model are stated in Section 3.

Figure 1. Analysis model, with each geometry point labeled with its coordinates

# Limit Equilibrium Analysis

The stability of the section was evaluated by the method of slices. The factor of safety is the ratio of the shear strength available along the failure surface to the shear required along it to hold the sliding mass in static equilibrium.

## Analysis Inputs

The problem inputs specific to the limit equilibrium model are shown in Figure 2, including the section and its materials, and the starting circle for the search.

Figure 2. Limit equilibrium model

| Method reported in detail | Spencer's Method |
| --- | --- |
| Slices | 40 |
| Surface family | circular |
| Starting circle | center (20, 40), R = 40 |
| Surface | located by search |
| Seismic coefficient | none |
| Maximum surface depth (elevation) | -10 |

## Materials

The material properties associated with the limit equilibrium analysis are shown in Table 1, including the strength option each material is analyzed under, the properties that option uses, and how its pore pressure is taken.

Table 1. Material properties

| # | Material | γ (pcf) | Strength | c (psf) | φ (deg) | Pore pressure |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | embankment | 130.0 | MC | 400.0 | 0.0 | none |
| 2 | foundation | 135.0 | MC | 800.0 | 0.0 | none |

The model defines no groundwater and no external water; the section is analyzed dry, with zero pore pressure throughout.

## Loads

The model carries no distributed loads.

## Spencer's Method

Spencer's Method is a complete-equilibrium procedure: it satisfies force equilibrium — both horizontal and vertical — and moment equilibrium of the sliding mass. Its single assumption is that the interslice forces are parallel, at one constant inclination θ, and the factor of safety and θ are solved together from those two conditions. The method applies to circular and non-circular surfaces and is the recommended basis for design.

### Search for the Critical Surface

The critical surface was located by automated search, which refines a grid of trial surfaces until the factor of safety stops improving. The reported surface, highlighted in Figure 3, is the lowest the search reached. Every trial circle evaluated is drawn in gray, and its center marked by a black dot, so the dots map out the grid of centers the search covered; the critical circle and its center are drawn in red. The green arrows run from each refinement stage's best center to the next, and shorten as the grid is refined about the minimum.

| Surface family | circular |
| --- | --- |
| Method | Spencer's Method |
| Trial surfaces evaluated | 529 |
| Grid centers refined | 62 |
| Refinement stages | 7 |
| Factor of safety range | 1.244 to 1.739 |
| Search window | none declared; the search was unconstrained |

Figure 3. Trial surfaces evaluated by the search, with the critical surface highlighted

### Results

Spencer's Method gives a factor of safety of 1.244 on the critical surface of Figure 4.

The solution reported the following admissibility notes, where the computed stresses depart from what the method assumes:

interslice tension on 15% of interior boundaries (min Z = -1436.3 vs max compression 7382.1)

line of thrust outside the slice on 15% of boundaries

Tension on the base of 2 slices near the crest. Consider adding a tension crack.

Figure 4. Critical surface and slice forces — Spencer's Method

### Slice Table

The geometry, forces and strengths of every slice on the critical surface as solved by Spencer's Method are listed in Table 2, numbered as in Figure 5. Forces are per unit thickness of section.

Figure 5. Slice numbering for the table below — Spencer's Method

Table 2. Slice data — Spencer's Method

| Slice | xc (ft) | yb (ft) | yt (ft) | Δx (ft) | Δl (ft) | α (deg) | W (lb/ft) | u (psf) | Mat | c (psf) | φ (deg) | N' (lb/ft) | Z (lb/ft) | Fh (lb/ft) | Fv (lb/ft) | Qs (lb/ft) | yQ (ft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5.19 | 2.07 | 2.60 | 1.27 | 1.33 | -17.71 | 86.2 | 0.0 | 1 | 400.0 | 0.00 | 305.2 | 0.0 | 0.0 | -86.2 | -505.4 | 2.07 |
| 2 | 6.46 | 1.69 | 3.23 | 1.27 | 1.32 | -15.98 | 253.4 | 0.0 | 1 | 400.0 | 0.00 | 467.9 | 505.4 | 0.0 | -253.4 | -541.6 | 1.69 |
| 3 | 7.72 | 1.35 | 3.86 | 1.27 | 1.31 | -14.26 | 413.8 | 0.0 | 1 | 400.0 | 0.00 | 619.7 | 1047.0 | 0.0 | -413.8 | -565.8 | 1.35 |
| 4 | 8.99 | 1.05 | 4.50 | 1.27 | 1.30 | -12.55 | 567.5 | 0.0 | 1 | 400.0 | 0.00 | 761.8 | 1612.8 | 0.0 | -567.5 | -578.8 | 1.05 |
| 5 | 10.26 | 0.78 | 5.13 | 1.27 | 1.29 | -10.86 | 714.8 | 0.0 | 1 | 400.0 | 0.00 | 894.8 | 2191.6 | 0.0 | -714.8 | -581.9 | 0.78 |
| 6 | 11.52 | 0.56 | 5.76 | 1.27 | 1.28 | -9.18 | 855.8 | 0.0 | 1 | 400.0 | 0.00 | 1019.5 | 2773.5 | 0.0 | -855.8 | -575.8 | 0.56 |
| 7 | 12.79 | 0.37 | 6.39 | 1.27 | 1.28 | -7.50 | 990.4 | 0.0 | 1 | 400.0 | 0.00 | 1136.6 | 3349.4 | 0.0 | -990.4 | -561.5 | 0.37 |
| 8 | 14.05 | 0.23 | 7.03 | 1.27 | 1.27 | -5.83 | 1118.9 | 0.0 | 1 | 400.0 | 0.00 | 1246.5 | 3910.9 | 0.0 | -1118.9 | -539.5 | 0.23 |
| 9 | 15.32 | 0.12 | 7.66 | 1.27 | 1.27 | -4.17 | 1241.2 | 0.0 | 1 | 400.0 | 0.00 | 1349.8 | 4450.4 | 0.0 | -1241.2 | -510.7 | 0.12 |
| 10 | 16.58 | 0.04 | 8.29 | 1.27 | 1.27 | -2.51 | 1357.5 | 0.0 | 1 | 400.0 | 0.00 | 1446.9 | 4961.1 | 0.0 | -1357.5 | -475.5 | 0.04 |
| 11 | 17.85 | 0.00 | 8.93 | 1.27 | 1.27 | -0.85 | 1467.8 | 0.0 | 1 | 400.0 | 0.00 | 1538.1 | 5436.5 | 0.0 | -1467.8 | -434.5 | 0.00 |
| 12 | 19.12 | 0.00 | 9.56 | 1.27 | 1.27 | 0.81 | 1572.0 | 0.0 | 1 | 400.0 | 0.00 | 1623.7 | 5871.1 | 0.0 | -1572.0 | -388.3 | 0.00 |
| 13 | 20.38 | 0.04 | 10.19 | 1.27 | 1.27 | 2.47 | 1670.1 | 0.0 | 1 | 400.0 | 0.00 | 1704.0 | 6259.4 | 0.0 | -1670.1 | -337.3 | 0.04 |
| 14 | 21.65 | 0.11 | 10.82 | 1.27 | 1.27 | 4.13 | 1762.3 | 0.0 | 1 | 400.0 | 0.00 | 1779.1 | 6596.7 | 0.0 | -1762.3 | -282.0 | 0.11 |
| 15 | 22.91 | 0.22 | 11.46 | 1.27 | 1.27 | 5.79 | 1848.3 | 0.0 | 1 | 400.0 | 0.00 | 1849.4 | 6878.8 | 0.0 | -1848.3 | -222.8 | 0.22 |
| 16 | 24.18 | 0.37 | 12.09 | 1.27 | 1.28 | 7.46 | 1928.3 | 0.0 | 1 | 400.0 | 0.00 | 1914.8 | 7101.6 | 0.0 | -1928.3 | -160.1 | 0.37 |
| 17 | 25.44 | 0.55 | 12.72 | 1.27 | 1.28 | 9.13 | 2002.1 | 0.0 | 1 | 400.0 | 0.00 | 1975.6 | 7261.7 | 0.0 | -2002.1 | -94.4 | 0.55 |
| 18 | 26.71 | 0.78 | 13.36 | 1.27 | 1.29 | 10.82 | 2069.6 | 0.0 | 1 | 400.0 | 0.00 | 2031.7 | 7356.1 | 0.0 | -2069.6 | -25.9 | 0.78 |
| 19 | 27.98 | 1.04 | 13.99 | 1.27 | 1.30 | 12.51 | 2130.7 | 0.0 | 1 | 400.0 | 0.00 | 2083.3 | 7382.1 | 0.0 | -2130.7 | 44.8 | 1.04 |
| 20 | 29.24 | 1.34 | 14.62 | 1.27 | 1.31 | 14.21 | 2185.4 | 0.0 | 1 | 400.0 | 0.00 | 2130.2 | 7337.3 | 0.0 | -2185.4 | 117.4 | 1.34 |
| 21 | 30.51 | 1.68 | 15.25 | 1.27 | 1.32 | 15.93 | 2233.4 | 0.0 | 1 | 400.0 | 0.00 | 2172.5 | 7219.9 | 0.0 | -2233.4 | 191.4 | 1.68 |
| 22 | 31.77 | 2.06 | 15.89 | 1.27 | 1.33 | 17.66 | 2274.7 | 0.0 | 1 | 400.0 | 0.00 | 2210.0 | 7028.6 | 0.0 | -2274.7 | 266.4 | 2.06 |
| 23 | 33.04 | 2.49 | 16.52 | 1.27 | 1.34 | 19.41 | 2309.0 | 0.0 | 1 | 400.0 | 0.00 | 2242.6 | 6762.1 | 0.0 | -2309.0 | 342.0 | 2.49 |
| 24 | 34.30 | 2.95 | 17.15 | 1.27 | 1.36 | 21.18 | 2336.1 | 0.0 | 1 | 400.0 | 0.00 | 2270.1 | 6420.1 | 0.0 | -2336.1 | 417.7 | 2.95 |
| 25 | 35.57 | 3.47 | 17.79 | 1.27 | 1.37 | 22.97 | 2355.8 | 0.0 | 1 | 400.0 | 0.00 | 2292.3 | 6002.4 | 0.0 | -2355.8 | 492.9 | 3.47 |
| 26 | 36.84 | 4.03 | 18.42 | 1.27 | 1.39 | 24.78 | 2367.8 | 0.0 | 1 | 400.0 | 0.00 | 2308.9 | 5509.5 | 0.0 | -2367.8 | 566.9 | 4.03 |
| 27 | 38.10 | 4.64 | 19.05 | 1.27 | 1.42 | 26.62 | 2371.7 | 0.0 | 1 | 400.0 | 0.00 | 2319.3 | 4942.6 | 0.0 | -2371.7 | 639.2 | 4.64 |
| 28 | 39.37 | 5.30 | 19.68 | 1.27 | 1.44 | 28.49 | 2367.2 | 0.0 | 1 | 400.0 | 0.00 | 2323.0 | 4303.4 | 0.0 | -2367.2 | 708.8 | 5.30 |
| 29 | 40.64 | 6.01 | 20.00 | 1.27 | 1.47 | 30.39 | 2309.4 | 0.0 | 1 | 400.0 | 0.00 | 2271.6 | 3594.6 | 0.0 | -2309.4 | 749.1 | 6.01 |
| 30 | 41.91 | 6.79 | 20.00 | 1.27 | 1.50 | 32.34 | 2181.6 | 0.0 | 1 | 400.0 | 0.00 | 2145.6 | 2845.5 | 0.0 | -2181.6 | 747.7 | 6.79 |
| 31 | 43.18 | 7.62 | 20.00 | 1.27 | 1.54 | 34.33 | 2043.6 | 0.0 | 1 | 400.0 | 0.00 | 2006.4 | 2097.8 | 0.0 | -2043.6 | 731.3 | 7.62 |
| 32 | 44.45 | 8.52 | 20.00 | 1.27 | 1.58 | 36.37 | 1894.9 | 0.0 | 1 | 400.0 | 0.00 | 1852.0 | 1366.5 | 0.0 | -1894.9 | 697.6 | 8.52 |
| 33 | 45.72 | 9.50 | 20.00 | 1.27 | 1.62 | 38.47 | 1734.4 | 0.0 | 1 | 400.0 | 0.00 | 1679.6 | 668.9 | 0.0 | -1734.4 | 643.5 | 9.50 |
| 34 | 46.99 | 10.54 | 20.00 | 1.27 | 1.67 | 40.62 | 1561.3 | 0.0 | 1 | 400.0 | 0.00 | 1485.7 | 25.4 | 0.0 | -1561.3 | 565.2 | 10.54 |
| 35 | 48.26 | 11.68 | 20.00 | 1.27 | 1.73 | 42.85 | 1374.2 | 0.0 | 1 | 400.0 | 0.00 | 1265.7 | -539.8 | 0.0 | -1374.2 | 457.5 | 11.68 |
| 36 | 49.53 | 12.90 | 20.00 | 1.27 | 1.80 | 45.17 | 1171.6 | 0.0 | 1 | 400.0 | 0.00 | 1013.5 | -997.3 | 0.0 | -1171.6 | 313.8 | 12.90 |
| 37 | 50.80 | 14.24 | 20.00 | 1.27 | 1.88 | 47.58 | 951.6 | 0.0 | 1 | 400.0 | 0.00 | 720.9 | -1311.1 | 0.0 | -951.6 | 125.2 | 14.24 |
| 38 | 52.07 | 15.69 | 20.00 | 1.27 | 1.98 | 50.10 | 711.7 | 0.0 | 1 | 400.0 | 0.00 | 376.0 | -1436.3 | 0.0 | -711.7 | -121.2 | 15.69 |
| 39 | 53.34 | 17.28 | 20.00 | 1.27 | 2.10 | 52.77 | 448.7 | 0.0 | 1 | 400.0 | 0.00 | -38.5 | -1315.1 | 0.0 | -448.7 | -443.9 | 17.28 |
| 40 | 54.61 | 19.04 | 20.00 | 1.27 | 2.25 | 55.62 | 158.0 | 0.0 | 1 | 400.0 | 0.00 | -549.4 | -871.2 | 0.0 | -158.0 | -871.2 | 19.04 |
| Total |  |  |  | 50.68 | 57.79 |  | 61392.8 |  |  |  |  | 60246.4 |  |  |  | -0.0 |  |

Slice: Slice number, counted from the left edge of the failure surface.   xc (ft): Horizontal coordinate of the slice mid-point.   yb (ft): Elevation of the failure surface at the slice mid-point.   yt (ft): Elevation of the ground surface at the slice mid-point.   Δx (ft): Slice width.   Δl (ft): Length of the slice base along the failure surface.   α (deg): Inclination of the slice base, positive when the base rises toward the toe.   W (lb/ft): Weight of the slice, per unit thickness.   u (psf): Pore water pressure on the slice base at its mid-point.   Mat: Index of the material the slice base runs through, numbered as on the materials sheet.   c (psf): Cohesion of the base material, in the strength formulation the analysis is run under.   φ (deg): Friction angle of the base material.   N' (lb/ft): Effective normal force on the slice base, per unit thickness, as solved.   Z (lb/ft): Interslice force on the right-hand side of the slice, per unit thickness.   Fh (lb/ft): Resultant of all the forces on the slice except the base normal, the base shear and the interslice forces — horizontal component, per unit thickness.   Fv (lb/ft): Resultant of all the forces on the slice except the base normal, the base shear and the interslice forces — vertical component, per unit thickness.   Qs (lb/ft): Resultant of the interslice forces on the slice (Spencer's Q), per unit thickness.   yQ (ft): Elevation of the line of action of Qs.

### Calculations

Figure 6. Forces on a slice — Spencer's Method

The equations below are in the symbols of the derivation published for Spencer's Method in the XSLOPE documentation; the numbers in them are the converged values.

Spencer's method lumps the interslice forces on each slice into a single resultant Q acting at the constant inclination θ, so that force and moment equilibrium of the whole sliding mass are two equations in the two unknowns F and θ. Fh and Fv are the sums of the forces on the slice of Figure 6 other than the base normal, the base shear and the interslice forces, as equations (1) and (2) of the derivation write them:

Fh = −kW − V + P sin β + T cos β + R cos ψ + H cos θp + L cos δ

Fv = −W − P cos β + T sin β + R sin ψ + H sin θp + L sin δ

The model carries no distributed load, seismic load, tension-crack water force, reinforcement crossing the failure surface, pile force and line load, and no shear on the top of the slice is simulated, so equations (1) and (2) reduce to:

Fh = 0

Fv = −W

Equations (23) and (24) give Q on each slice from them and from the strength mobilized on its base:

Q = [−Fv·sin α − Fh·cos α − c·ΔlF + (Fv·cos α − Fh·sin α + u·Δl)·tan φF]·mα

mα = 1cos (α − θ) + sin (α − θ)·tan φF

There is no equation for F: it appears inside Q, in the mobilized cohesion c·Δl/F and the friction tan φ/F, and again with θ in mα. A trial pair (F, θ) fixes Q on every slice.

Fh, Fv, Q and yQ are columns Fh, Fv, Qs and yQ of the slice table; each row's Q follows from that row through the two equations above, at the converged F and θ.

Substituting Q and yQ into the equilibrium of the whole sliding mass gives equations (27) and (28) of the derivation, two equations in the two unknowns. R1 is the force imbalance and R2 the moment imbalance, and the solution is the pair (F, θ) at which both are zero:

R1 = Q

R2 = Q·(xb·sin θ − yQ·cos θ)

F and θ are iterated together: each trial pair recomputes both sums, and the pair is adjusted until both vanish.

The symbols used above are defined in Table 3. Those that are columns of Table 2 carry a value for every slice.

Table 3. Nomenclature — Spencer's Method

| Symbol | Meaning |
| --- | --- |
| T | shear force on the top of the slice |
| P | resultant of the distributed load on the top of the slice, in the symbols of equations (1) and (2) — column D of the slice table, and written D below, where P is instead the reinforcement force |
| Δl (ft) | Length of the slice base along the failure surface |
| α (deg) | Inclination of the slice base, positive when the base rises toward the toe |
| W (lb/ft) | Weight of the slice, per unit thickness |
| u (psf) | Pore water pressure on the slice base at its mid-point |
| c (psf) | Cohesion of the base material, in the strength formulation the analysis is run under |
| φ (deg) | Friction angle of the base material |
| Fh (lb/ft) | Resultant of all the forces on the slice except the base normal, the base shear and the interslice forces — horizontal component, per unit thickness |
| Fv (lb/ft) | Resultant of all the forces on the slice except the base normal, the base shear and the interslice forces — vertical component, per unit thickness |
| yQ (ft) | Elevation of the line of action of Qs |
| F | factor of safety |
| β | inclination of the distributed load from vertical (perpendicular to the slope) |
| ψ | angle of the reinforcement force from horizontal |
| θp | angle of the pile force from horizontal (positive counterclockwise, upward) |
| δ | angle of the line load from horizontal (−90° is straight down) |
| θ | inclination of the interslice forces from horizontal |
| mα | the base-normal denominator, which carries the factor of safety and is what makes the solution iterative |
| L | line load applied on top of the slice, per unit thickness |
| kW | horizontal seismic force on the slice, per unit thickness — column kW of the slice table |
| R | reinforcement force on the slice base, at the angle ψ from horizontal — column P of the slice table, and written P below |
| V | resultant force of the water in a tension crack — column Tc of the slice table, which the other methods' equations write T |
| H | pile or pier force on the slice at the failure surface — column Hp of the slice table |
| Q | resultant of the interslice forces on the slice, at the inclination θ — column Qs of the slice table |
| xb | horizontal coordinate of the slice base mid-point |
| R1 | force imbalance of the whole sliding mass at a trial (F, θ) — zero at the solution |
| R2 | moment imbalance of the whole sliding mass at a trial (F, θ) — zero at the solution |

The iteration converged at F = 1.244 with θ = 8.48 degrees. The force equation is the Qs column of Table 2 added up over the 40 slices, printed as the total at the foot of that column:

R1 = Q = -5.097e-07

That is one part in 35 billion of the 17636.7 the magnitudes of Q come to. The moment equation weights the same column by xc — the xb of equation (28) — and by yQ, at the θ above:

R2 = Q·(xb·sin θ − yQ·cos θ) = 1.641e-05

That is one part in three billion of the 43715.5 its moment terms come to. Sums re-formed from the printed, rounded columns carry that rounding and close to within a thousandth of these totals.